SIH 2026 — GEO-CODED IMAGERY WORK PACKAGE

Shiggaon Taluk, Haveri District, Karnataka | Standalone Teammate README

Purpose: This is the execution guide for the teammate responsible for the Geo-Coded Imagery component. It is designed to be followed independently and to produce outputs that can later plug directly into the main QGIS/GIS project.

Pilot area: Shiggaon Taluk, Haveri district, Karnataka. Spatial integration unit: the project's 66 micro-watersheds. Master integration key: mws_id.

QGIS target: QGIS Desktop 3.44.12. Main project analysis CRS: WGS 84 / UTM Zone 43N (EPSG:32643). Standard EXIF GPS coordinates are normally read as WGS 84 geographic coordinates (EPSG:4326) unless the source metadata documents otherwise.

Critical rule: Never invent, guess, or silently move coordinates. Preserve original photographs and their original metadata. Every derived GIS point must remain traceable to its source image.

# 1. Objective

Turn real-world field photographs into structured spatial evidence. A successful record answers: WHERE was it taken, WHEN, WHAT is visible, WHICH micro-watershed contains it, and HOW can the evidence support watershed interpretation or later decision-making?

FIELD PHOTO
   ↓
Original image + EXIF
   ↓
GPS/date extraction
   ↓
Coordinate QA
   ↓
GIS point
   ↓
Point-in-polygon → mws_id
   ↓
Observation + evidence metadata
   ↓
QGIS map / later LULC + risk integration

# 2. Success criteria

Every usable photo has a unique image_id.

Original images remain unchanged in Raw_Photos.

Latitude/longitude and capture time are retained when available.

Photo points load correctly in QGIS.

Every in-area point receives the correct mws_id from the authoritative 66-MWS layer.

Missing, suspicious, outside-area, and boundary cases are explicitly flagged.

Every point is traceable to its original filename and image.

Outputs are delivered as both a structured metadata table and a GIS layer.

All processing and corrections are documented.

# 3. Folder structure

SIH_2026_SHIGGAON/
└── 12_GEO_CODED_IMAGES/
    ├── Raw_Photos/
    ├── Processed_Photos/
    ├── Metadata/
    └── GIS_Points/

Raw_Photos = untouched originals. Processed_Photos = presentation/derived copies only. Metadata = CSV/XLSX inventory, QA and logs. GIS_Points = final GeoPackage/CSV spatial outputs.

# 4. Coordination with the main GIS teammate

Use the same 66-micro-watershed dataset as the main project.

Use its existing mws_id values exactly; do not create alternative IDs.

Do not edit, clip, simplify, or otherwise modify the authoritative MWS polygons.

Use the same Shiggaon study-area definition as the main GIS project.

Keep the imagery workflow independent until the handover; then load the final GeoPackage into the master QGIS project.

# 5. Field photo collection

Enable location services before field capture.

Ensure the camera app has permission to record location.

Capture only after the device has a reliable location fix.

Prefer locations with reasonable GNSS visibility.

Keep capture date/time enabled.

Take contextual photos that clearly show the watershed feature.

Where a site is important, take representative photos rather than guessing a coordinate.

Do not assume a displayed map pin is embedded in the image EXIF; verify the actual metadata.

# 6. Suggested observation categories

| Category | Examples | Later use |
| --- | --- | --- |
| Water body | pond, tank, reservoir, wetland | water evidence/context |
| Drainage / stream | channel, obstruction, erosion | hydrology context |
| Spring / recharge | spring, seepage, recharge-area condition | spring/recharge context |
| Agriculture | crop, irrigation, exposed field | LULC/agriculture context |
| Vegetation | vegetation, degraded vegetation, crop canopy | NDVI/LULC context |
| Soil / erosion | bare soil, gully, sediment | soil/erosion evidence |
| Built-up | settlement, road, construction near drainage | land-pressure context |
| Intervention | check dam, farm pond, restoration measure | intervention evidence |
| Other | important watershed observation | explicit notes |

# 7. Master metadata table

Create Photo_Master.csv or Photo_Master.xlsx in 12_GEO_CODED_IMAGES/Metadata/. One row represents one photograph.

| Field | Type | Required | Meaning |
| --- | --- | --- | --- |
| image_id | text | YES | Permanent unique project identifier, e.g. IMG_0001 |
| file_name | text | YES | Original filename |
| file_path | text | YES | Relative path to source image |
| latitude | decimal | For geotagged | EXIF latitude in decimal degrees |
| longitude | decimal | For geotagged | EXIF longitude in decimal degrees |
| capture_datetime | datetime | Preferred | Capture date/time; document timezone status |
| gps_source | text | YES | EXIF / survey / other documented source |
| gps_accuracy_m | decimal | If available | Reported device/GNSS accuracy; never invent |
| coordinate_status | text | YES | VALID / MISSING / SUSPICIOUS / OUTSIDE_STUDY_AREA |
| mws_id | text | In-area points | Assigned by spatial join |
| study_area_status | text | YES | INSIDE / BOUNDARY_REVIEW / OUTSIDE |
| observation_type | text | YES | Water body / stream / spring / etc. |
| description | text | YES | Short factual description |
| photo_quality | text | YES | GOOD / ACCEPTABLE / POOR |
| evidence_relevance | text | YES | HIGH / MEDIUM / LOW |
| intervention_type | text | If applicable | Observed intervention, if any |
| remarks | text | Optional | Review notes/exceptions |
| review_status | text | YES | PENDING / VERIFIED / REJECTED |
| reviewer | text | Optional | Reviewer name/initials |

# 8. image_id standard

IMG_0001
IMG_0002
IMG_0003
...
IMG_9999

image_id is the stable project key. Keep the original file_name separately. Do not change image_id after the record is integrated.

# 9. EXIF extraction

Read the original EXIF metadata; do not modify the originals. Extract latitude, longitude and capture datetime when available. If coordinates are stored as degrees/minutes/seconds, convert to decimal degrees for GIS while retaining the original representation when practical.

Decimal latitude  = degrees + minutes/60 + seconds/3600
Decimal longitude = degrees + minutes/60 + seconds/3600
Apply the correct sign for South/West coordinates.

Do not use a guessed geographic range as proof of correctness. Spatially validate the point against the Shiggaon study framework and map context.

# 10. Coordinate QA/QC

Check that latitude and longitude are numeric.

Check valid numeric ranges: latitude -90 to +90; longitude -180 to +180.

Flag 0,0 or other obviously invalid coordinates.

Check duplicate coordinates; multiple photos at one site can be legitimate.

Plot the point and verify that it falls in the expected region.

Determine whether it is inside, boundary-related, or outside the study area.

Visually inspect suspicious points against available map/imagery context.

Never move a point just to make it fit a polygon.

Only create a corrected coordinate when an independent, documented source exists.

Record the reason for every correction, rejection, or boundary decision.

# 11. Missing GPS

Missing GPS is not the same as invalid GPS.

| Status | Meaning |
| --- | --- |
| MISSING | No usable coordinate found in EXIF |
| SUSPICIOUS | Coordinate exists but fails spatial/metadata review |
| OUTSIDE_STUDY_AREA | Coordinate is valid but outside the defined pilot area |
| VALID | Coordinate passes basic and spatial review |
| BOUNDARY_REVIEW | Point is on/near a micro-watershed boundary and needs review |

Never invent coordinates for a photo without GPS. If a separately surveyed coordinate exists, document it as a separate gps_source and preserve the fact that EXIF GPS was missing.

# 12. Create the QGIS point layer

Open QGIS Desktop 3.44.12.

Add the validated CSV as a Delimited Text layer.

Use longitude as X field and latitude as Y field.

Use EPSG:4326 for standard WGS 84 EXIF latitude/longitude unless the source metadata documents another CRS.

Load and visually inspect the points.

Save a persistent copy in 12_GEO_CODED_IMAGES/GIS_Points/ as a GeoPackage layer.

Use the layer name GEO_CODED_PHOTO_POINTS.

Keep the CSV as the audit/source table.

# 13. Assign mws_id by point-in-polygon

This is the key integration operation.

PHOTO POINTS
     +
AUTHORITATIVE 66-MWS POLYGONS
     ↓
POINT-IN-POLYGON / JOIN ATTRIBUTES BY LOCATION
     ↓
PHOTO POINTS WITH mws_id

Load the authoritative 66-MWS layer.

Confirm the exact mws_id field with the main GIS teammate.

Confirm both layers align spatially.

Use QGIS Processing → Join attributes by location.

Target layer = photo points.

Join layer = 66 micro-watersheds.

Use the point-within-polygon relationship.

Preserve all photo points so unmatched/outside records remain visible for QA.

Check that each in-area point receives one expected mws_id.

Save the joined result as GEO_CODED_PHOTO_POINTS_MWS.gpkg.

# 14. Boundary cases

Flag points that fall exactly on or extremely close to an MWS boundary.

Do not randomly assign a boundary point.

Do not edit the MWS polygon to accommodate a photo.

Use documented geometry/field evidence if a boundary case must be resolved.

Keep a boundary_review/remarks field so the decision is auditable.

# 15. Final spatial fields

| Field | Purpose |
| --- | --- |
| image_id | Photo database key |
| file_name | Original file traceability |
| latitude / longitude | Original GPS position |
| capture_datetime | Time evidence was captured |
| mws_id | Common watershed integration key |
| observation_type | Field evidence class |
| description | Factual description |
| coordinate_status | Coordinate QA |
| study_area_status | Spatial relationship |
| photo_quality | Image quality |
| evidence_relevance | Project usefulness |
| review_status | QA state |
| remarks | Exceptions/comments |

# 16. Integration with the main GIS

SHIGGAON GIS
                       │
               66 MICRO-WATERSHEDS
                       │
                     mws_id
                       │
       ┌───────────────┼────────────────┐
       ↓               ↓                ↓
      LULC          Terrain          Hydrology
       │               │                │
       └───────────────┼────────────────┘
                       ↓
                  MODEL 2 RISK
                       ↑
                       │
                GEO-CODED PHOTO
                image_id + mws_id

The photo layer is evidence/context. It is not a replacement for raster or model analysis. Later, the main team can summarize photo evidence by mws_id and compare field observations with LULC, NDVI/NDWI, terrain, hydrology and Model 2 outputs.

# 17. Optional later comparison fields

| Field | Use |
| --- | --- |
| lulc_context | Compare observed field context with Model 1 |
| water_presence | Contextual comparison with NDWI/water mapping |
| vegetation_context | Contextual comparison with NDVI |
| erosion_observed | Field evidence for erosion/soil condition |
| drainage_condition | Observed stream/channel condition |
| intervention_observed | Observed watershed intervention |
| risk_context | Compare with Model 2; do not manually invent model risk |
| primary_driver_context | Compare with Model 2; clearly distinguish observed vs model-derived |
| intervention_recommendation | Only after the project team defines a decision rule |

# 18. Image storage rules

Raw_Photos contains originals only.

Processed_Photos contains copies for presentation/processing.

Never overwrite the original.

Keep image_id in derived filenames where useful, e.g. IMG_0001_preview.jpg.

Do not store the only copy of a photo inside a QGIS project.

# 19. QGIS presentation

For the final demonstration, use a clear photo-point symbol and an identification popup/form. Show image_id, observation_type, description, capture_datetime, mws_id, coordinate_status and photo_quality. Keep the map readable rather than exposing every metadata field.

# 20. Mandatory QA checklist

[ ] Original photos preserved.

[ ] Master metadata table exists.

[ ] image_id unique.

[ ] Original filename retained.

[ ] GPS latitude/longitude extracted where available.

[ ] GPS source recorded.

[ ] Missing GPS explicitly flagged.

[ ] Suspicious/outside/boundary cases flagged.

[ ] Point layer loads correctly.

[ ] Source CRS documented.

[ ] Authoritative 66-MWS layer used.

[ ] mws_id values preserved exactly.

[ ] In-area points assigned mws_id or explicitly flagged.

[ ] No MWS polygon modified.

[ ] GeoPackage output exists.

[ ] CSV metadata output exists.

[ ] QA log exists.

[ ] Processing log exists.

[ ] Sample points visually inspected.

# 21. Required handover files

| Deliverable | Location / filename |
| --- | --- |
| Validated photo metadata | 12_GEO_CODED_IMAGES/Metadata/Photo_Master_Validated.csv |
| Spatial photo points | 12_GEO_CODED_IMAGES/GIS_Points/GEO_CODED_PHOTO_POINTS.gpkg |
| Spatial points with MWS | 12_GEO_CODED_IMAGES/GIS_Points/GEO_CODED_PHOTO_POINTS_MWS.gpkg |
| Original photos | 12_GEO_CODED_IMAGES/Raw_Photos/ |
| Processed copies | 12_GEO_CODED_IMAGES/Processed_Photos/ |
| QA log | 12_GEO_CODED_IMAGES/Metadata/GeoCoded_QA_Log.xlsx |
| Processing log | 12_GEO_CODED_IMAGES/Metadata/GeoCoded_Processing_Log.txt |

# 22. Processing log template

DATE:
OPERATOR:
QGIS VERSION: 3.44.12

INPUT PHOTO COUNT:
GEOTAGGED PHOTO COUNT:
MISSING-GPS COUNT:
SUSPICIOUS-GPS COUNT:
OUTSIDE-STUDY-AREA COUNT:
BOUNDARY-REVIEW COUNT:
VALID POINT COUNT:

POINT CREATION METHOD:
SOURCE CRS:
MWS LAYER USED:
MWS ID FIELD:
SPATIAL JOIN METHOD:

OUTPUT LAYER:
ISSUES FOUND:
CORRECTIONS MADE:
REJECTED/FLAGGED RECORDS:
QA SAMPLE SIZE:
QA RESULT:

HANDOVER DATE:

# 23. Suggested execution schedule

| Stage | Work | Output |
| --- | --- | --- |
| 1 | Inventory photos; establish folder discipline | Photo inventory |
| 2 | Extract EXIF GPS/date metadata | Photo_Master |
| 3 | Coordinate QA and visual review | Validated metadata |
| 4 | Create QGIS photo points | GEO_CODED_PHOTO_POINTS |
| 5 | Join authoritative 66 MWS using mws_id | GEO_CODED_PHOTO_POINTS_MWS |
| 6 | Review boundary/outside cases | QA-clean spatial layer |
| 7 | Prepare popup/presentation and documentation | Handover package |

# 24. Do NOT do these

Do not invent GPS coordinates.

Do not silently correct coordinates.

Do not edit the authoritative 66-MWS polygons.

Do not create alternate watershed IDs.

Do not delete missing-GPS originals.

Do not overwrite original photos.

Do not mix observed field information with Model 2 predictions without labeling the distinction.

Do not claim a photo proves a model output without a proper validation method.

Do not change the main GIS project's CRS just to accommodate photo coordinates.

# 25. Final handover message template

Geo-coded imagery work completed.

Files:
1. Photo_Master_Validated.csv
2. GEO_CODED_PHOTO_POINTS_MWS.gpkg
3. GeoCoded_QA_Log.xlsx
4. GeoCoded_Processing_Log.txt
5. Raw_Photos/

Summary:
Total photos:
Geotagged:
Missing GPS:
Suspicious:
Outside study area:
Boundary review:
Valid spatial points:
Points assigned to mws_id:

MWS layer used:
mws_id field:
Source CRS:
Output CRS:

Known limitations:
...

Ready for integration into the main Shiggaon QGIS project.

End of README. Any deviation from this workflow should be recorded in the processing log so the final SIH submission remains reproducible and auditable.